# Customer Release Notes

U.S. Robotics 8550 ADSL Ethernet modem

Firmware Version 3.1.0 (August 23, 2002)

# Introduction

Thank you for choosing U.S. Robotics’ ADSL Ethernet modem.

The following list outlines the three main sections of this document.  Each section contains important information about the USR8550 ADSL Ethernet modem and firmware version 3.1.0.

- Key Modifications

- Configuration Examples

- Known problems

# Key Modifications

## Improvements

DNS Relay is supported in PPPoA and embedded PPPoE mode.

A secondary DNS IP address is automatically stored and accessed when offered by your ISP’s DHCP server.

Single user PPTP pass-through session is supported

Support for ALG(s) (ICQ©, Net Meeting©)

## Web Graphical User Interface Changes

LAN Settings

The Modem’s Web Interface contains two new options – IP Filtering & MAC Translator.    Configuration of these options is not required.

IP Filtering – Used to setup remote management access to the modem via Telnet or Web GUI.  When this table is empty the default action is to block all access from the WAN port.  Unless authorized by your ISP it is not recommended that you configure this option.

MAC Translator – This table creates a binding between an existing bridge PVC and the Client’s Network Card’s MAC address.  Once configured, only the Ethernet frames belonging to the Source Mac Address in the table are forwarded to this PVC. When the table is empty the default action is to forward all Ethernet frames to all available PVC(s). Unless authorized by your ISP it is not recommended that you configure this option.

NAPT – Generic Routing Encapsulation GRE has been added to support PPTP pass-through.

# Configuration Notes

DNS Relay – No Configuration is required

Secondary DNS – No Configuration is required

Single User PPTP Pass-through in routed mode

(Note: you must be connected to your ISP and have valid public IP address)

Add NAPT entries follows:

- Interface – Select your public address

- Port Num – Leave Blank

- Protocol – GRE

- Server IP Address – Enter the IP address assigned to your computer by the U.S. Robotics ADSL Ethernet modem.

- Click Add and then OK

- Click on the Save Configuration Menu and save

- Reboot the modem

A PPTP tunnel can now originate on your computer to your ISP’s PPTP equipment

ALG support

The NAPT menu is used to create a connective pathway through Port Address Translation for applications that listen on a particular port, or embed IP header information in their data.  There may be security implications inherent in enabling ALG support, which must be weighed against the need for these specific applications.  ALG support should not be enabled unless there is a specific need for these applications.

Configure for Net Meeting

NAPT - add this table entry

- Choose you interface with the public address assigned by your ISP

- Port Num – 1720

- Protocol – TCP

- Server IP Address – Your computer’s IP address (assigned by the U.S. Robotics USR8550 ADSL Ethernet modem)

- Click OK and Save changes

Configure for ICQ

ICQ uses dynamic range of listening ports and should be configured for a specific port. For complete instructions visit ICQ website: http://www.icq.com/icqtour/firewall/.

NAPT - add this table entry

- Choose you interface with the public address assigned by your ISP

- Port Num – Enter the port number set in the ICQ Application. If the application is unable to connect on a single port then enter “all” as the port number

- Protocol – TCP

- Server IP Address – Your computer’s IP address (assigned by the U.S. Robotics USR8550 ADSL Ethernet modem)

- Click OK and Save changes

# Known Issues

- Only one PPTP tunnel is allowed in pass-through mode.