| SAMPLE PREVENTATIVE MAINTENANCE INSPECTION SHEET |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIXED ROUTE VEHICLE INSPECTION |  |  |  |  |  |  |  |  |
| Date:____________ Vehicle #:____________ Mileage:____________ Interval:______ Inspector:____________ |  |  |  |  |  |  |  |  |
| # TYPE OF OPERATIONS TO BE PERFORMED: '/' if Okay; 'X' if Adjusted; 'O' if Repairs are Required |  |  |  |  |  |  |  |  |
| PM WORK ORDER NUMBER:_________________ |  |  |  |  |  | REPAIR WORK ORDER NUMBER:______________ |  |  |
| OPERATIONAL CONTROLS |  |  |  | ## SURVEILLANCE SYSTEM |  |  |  |  |
|  |  | # Verify that starter will only work in Neutral |  |  |  | Check operation, adjustment, and damage on cameras |  |  |
|  |  | #### Check operations of wipers and washers |  |  |  | Inspect recorder securement, mounts, wiring, damage |  |  |
|  |  | Inspect all gauges for proper operation |  |  |  |  |  |  |
|  |  | Attempt to move bus with Park Brake applied |  | EXTERIOR |  |  |  |  |
|  |  | Check door control operation in all positions |  |  |  | Inspect reflectors for securement, damage, & wear |  |  |
|  |  | Check throttle interlock with exit door activated |  |  |  | Inspect panels and skirts for secure., damage, & wear |  |  |
|  |  | Check brake interlock with exit door activated |  |  |  | Steam clean engine and batteries |  |  |
|  |  | Check throttle interlock with kneeler activated |  |  |  | Gently steam clean radiator core (no caustic cleaners) |  |  |
|  |  | Check brake interlock with kneeler activated |  |  |  |  |  |  |
|  |  | Check operation of door dump valve |  | TIRES AND WHEELS |  |  |  |  |
|  |  | ### Trip door sens edge; should set alarm and open door |  |  |  | Inspect wheel hardware for loose lugs |  |  |
|  |  | Check climate control and reheat mode |  |  |  | Inspect tires for damage |  |  |
|  |  | Check climate control potentiometer |  |  |  | Check hubodometer for securement and damage |  |  |
|  |  | Check blower operation on all speeds |  |  |  | Check for missing valve caps and damaged stems |  |  |
|  |  | Verify proper blower air flow at Driver area ducting |  |  |  | Inspect wheels for damage, cracks, and rust trails |  |  |
|  |  | Operate kneeler. Check alarm and warning lights |  |  |  | Check oil level in front hub oil reservoir |  |  |
|  |  | Check fast idle. (switch must be off to move G-90) |  |  |  |  |  |  |
|  |  | Check all exterior lights for proper operation |  | BATTERIES |  |  |  |  |
|  |  | Check all interior lights. Lamps must cancel in Reverse |  |  |  | Remove corrosion and check for proper connections |  |  |
|  |  | Check operation of step well lights |  |  |  | Check cables and terminals for fraying and routing |  |  |
|  |  | Check operation of hazard lights and audible devices |  |  |  | Fill cells on non maintenance-free batteries |  |  |
|  |  | Check operation of signal lights and audible devices |  |  |  | Check battery hold downs for securement and cond. |  |  |
|  |  | Verify Low Air alarm and light activate @ 68-74 PSI |  |  |  | Check charge rate (27.8-28.2 VDC w/fast idle on) |  |  |
|  |  | Check operation of mirrors, heat, and dash indicators |  |  |  | Apply lube to battery tray slides |  |  |
|  |  | Check air buildup: 90-120 within 5 min w/fast idle on |  |  |  | Check alignment of battery compartment door |  |  |
|  |  | Check operation of horn |  |  |  | Inspect battery compartment door seals |  |  |
|  |  | ### Check operation of all indicator lamps |  |  |  |  |  |  |
|  |  | Check B/U lamps and warning alarm in Reverse |  | UNDERCARRIAGE |  |  |  |  |
|  |  | Test Destination Sign for proper operation |  |  |  | Check front end alignment |  |  |
|  |  | Check operation of P/A, mic, amp, and foot switch |  |  |  | Inspect suspension components for damage and wear |  |  |
|  |  | Inspect driver's seatback, bottom, lumbar, and mounts |  |  |  | Check for proper ride height |  |  |
|  |  | Inspect operation of driver's seatbelt |  |  |  | Inspect drag link and ends for damage and wear |  |  |
|  |  | Inspect the tilt/telescopic function of the column |  |  |  | Inspect tie rods for damage, wear, and securement |  |  |
|  |  | Test emergency alarm/destination sign activation |  |  |  | Inspect steering arms for damage and securement |  |  |
|  |  |  |  |  |  | Check kingpins for excessive movement |  |  |
| INTERIOR |  |  |  |  |  | Inspect swaybar for damage, loose/worn components |  |  |
|  |  | Inspect interior for damage, wear, and grafitti |  |  |  | Inspect air lines for chaffing, proper clamps/routing |  |  |
|  |  | Inspect flooring for damage and wear |  |  |  | Inspect steering gear for leaks, securement, & damage |  |  |
|  |  | Inspect glass for damage or grafitti |  |  |  | Inspect steering lines for leaks, damage, and routing |  |  |
|  |  | Check operation and condition of visors and shades |  |  |  | Inspect power steering pump operation and condition |  |  |
|  |  | Inspet step treads for damage and wear |  |  |  | Inspect steering column condition and operation |  |  |
|  |  | Check rails and stanchions for damage/securement |  |  |  | Inspect steering column u-joints for wear |  |  |
|  |  | Inspect w/c belts for operation and securement |  |  |  | Soap test air bags and inspect for leaks and condition |  |  |
|  |  | Verify there are two spare belts per vehicle |  |  |  | Inspect brake application valve for leaks and condition |  |  |
|  |  | Inspect emergency exits for operation and decals |  |  |  | Inspect brake chambers for leaks and condition |  |  |
|  |  | Lubricate driver's seat and check adjusters |  |  |  | Record lining depth. Inspect for looseness or damage |  |  |
|  |  | Inspect Accident/Vomit Kit for proper inventory |  |  |  | Inspect fuel tank straps for condition or damage |  |  |
|  |  | Check operation of window latches and slides |  |  |  | Inspect fuel lines for chaffing, clamps, and leaks |  |  |
|  |  | Check passenger seats for damage and grafitti |  |  |  | Inspect fuel cap for leaks and securement |  |  |
|  |  | Check chime system, dash indicator, & ADA chime |  |  |  | Inspect frame for cracks and damage |  |  |
|  |  | Inspect fire extinguisher charge, mount, & safety seal |  |  |  | Verify air governor cut-out @ 120# |  |  |
|  |  | Inspect First-Aid Kit for proper inventory |  |  |  | Inspect air brake lines for chaffing, clamps, and leaks |  |  |
|  |  | Check triangle kit for damaged components |  |  |  | Drain air tanks. Inspect for moisture and oil |  |  |
|  |  | Check air intake filters and grills; replace as needed |  |  |  | Inspect relay valves for leaks and damage |  |  |
|  | UNDERCARRIAGE (continued) |  |  |  | ELECTRONIC DIAGNOSTICS |  |  |  |
|  |  |  | check operation and condition of moisture ejector |  |  |  | If CEL on, use reader to retrieve codes |  |
|  |  |  | Inspect slack adjusters. Maximum stroke: 1.5-2.0" |  |  |  |  |  |
|  |  |  | Inspect pinion seals for leaks |  | RADIATOR |  |  |  |
|  |  |  | ### Inspect axle gaskets for leaks |  |  |  | Inspect for leaks, dirt, and debris |  |
|  |  |  | Inspect carrier to housing gasket for leaks |  |  |  | Inspect mounts and louvers for damage |  |
|  |  |  | Inspect radius rods for wear, damage, and securement |  |  |  | Inspect all skirts and panels for damage |  |
|  |  |  | Inspect lateral rod for wear, damage, and securement |  |  |  | Check surge tank, fill cap, and pressure cap |  |
|  |  |  | Inspect shocks and bushings for wear and damage |  |  |  |  |  |
|  |  |  | Inspect axle and pinion nuts for securement |  | TRANSMISSION |  |  |  |
|  |  |  | Inspect axle housing mounts for cracks and damage |  |  |  | Inspect case for leaks, damage, and securement |  |
|  |  |  | Inspect suspension mounts for cracks and damage |  |  |  | Inspect mounts for wear and damage |  |
|  |  |  | Inspect propeller shaft for wear and damage |  |  |  | Inspect lines for wear, leaks, and damage |  |
|  |  |  | Check u-joints for lock wire and tightness |  |  |  | Inspect transmission cooler for fluid leaks |  |
|  |  |  | Inspect leveling valves for adjustment and leaks |  |  |  | Inspet retarder housing for leaks and securement |  |
|  | LUBRICATION |  |  |  | BIKE RACK AND BIKE RACK AD SIGN |  |  |  |
|  |  |  | Lube all pivot points on bike rack |  |  |  | Inspect rack for damage and function |  |
|  |  |  | Lube exit door pins |  |  |  | Inspect latch for operation and damage |  |
|  |  |  | Lube windshield wiper pivot joints |  |  |  | Inspect brush guard |  |
|  |  |  | Lube throttle pivots (RTS) |  |  |  | Check condition of paint |  |
|  |  |  | Lube upper rollers on entrance and exit doors |  |  |  | Check securement of ad sign on bike rack |  |
|  |  |  | Lube steering tie-rod ends (2 zerks) |  |  |  |  |  |
|  |  |  | Lube drag link ends (2 zerks) |  | VERICOM BRAKE TEST |  |  |  |
|  |  |  | Lube kingpins (4 zerks) (raise axle to relieve pressure) |  |  |  | Test #1: |  |
|  |  |  | Lube driveline u-joints (2 zerks) |  |  |  | Test #2: |  |
|  |  |  | Lube driveline slip spline (1 zerk) |  |  |  | Test #3: |  |
|  |  |  | Lube steering column u-joints (4 zerks) |  |  |  |  |  |
|  |  |  | Lube engine door pivots |  | MOTOR GUARD SYSTEM |  |  |  |
|  |  |  | Lube s-cam tubes (1 zerk per wheel position) |  |  |  | Test coolant probe per manual |  |
|  |  |  | Lube slack adjusters (1 zerk per wheel position) |  |  |  | Test 'Hot Engine' shutdown per manual |  |
|  |  |  | Check oil level in differential. Change if contaminated |  |  |  | Test fire alarm per manual |  |
|  |  |  | Drain engine oil |  |  |  | Test 'Low Oil' shutdown per manual |  |
|  |  |  | Replace engine oil filters |  |  |  |  |  |
|  |  |  | Install and tighten engine oil drain plug |  | BATTERY LOAD TEST RESULTS (minimum 9.6 VDC) |  |  |  |
|  |  |  | Refill engine with 15/40 oil (40 wt on 2-cycle engines) |  |  |  | Battery #1: VDC |  |
|  |  |  |  |  |  |  | Battery #2: VDC |  |
|  | ENGINE COMPARTMENT |  |  |  |  |  | Battery #3: VDC |  |
|  |  |  | Inspect belts and automatic tensioners |  |  |  | Battery #4: VDC |  |
|  |  |  | Inspect engine for leaks and abnormal noises |  |  |  |  |  |
|  |  |  | Inspect hydraulic tank for proper level and leaks |  | WHEELCHAIR LIFT (except RTS) |  |  |  |
|  |  |  | Inspect transmission fluid level. Adjust as needed |  |  |  | Cycle lift and check for proper operation |  |
|  |  |  | Test antifreeze strength |  |  |  | Test sensitive edges and mats |  |
|  |  |  | Check coolant level in tank. Adjust as needed |  |  |  | Remove access panels and guards |  |
|  |  |  | Inspect air induction system for leaks and securement |  |  |  | Inspect entire structure for cracks and damage |  |
|  |  |  | Inspect air restriction indicator for flag or damage |  |  |  | Check for hydraulic leaks and chaffing hoses |  |
|  |  |  | Inspect engine mounts for wear and securement |  |  |  | Inspect electrical components for chaffing/D37damage |  |
|  |  |  | Inspect air compresor for air, oil, and coolant leaks |  |  |  | Inspect condition and adjustment of drive chains |  |
|  |  |  | Inspect air lines for leaks, chaffing, and clamps |  |  |  | Apply lubrication to all pivots and chains |  |
|  |  |  | Inspect charge air cooler for wear, damage, or leaks |  |  |  | Reinstall guards and pans |  |
|  |  |  | Drain water from fuel/water seperator |  |  |  | Inspect hydraulic pump unit for leaks and connections |  |
|  |  |  | Inspect hydraulic system for leaks |  |  |  | Check oil level in hydraulic unit. Adjust as needed |  |
|  |  |  | Inspect electrical wiring for chaffing and clamps |  |  |  | Recycle lift to ensure proper operation |  |
|  |  |  | Check operation of engine compartment lights |  |  |  |  |  |
|  |  |  | Inspect gauges and switches in rear-run control box |  | VENTILATION SYSTEM |  |  |  |
|  |  |  | Drain air box canister (2-cycle only) |  |  |  | Inspect ventilation filter. Replace as needed |  |

| PERFORMANCE TEST |  | TIRES-RECORD TREAD DEPTH AND PRESSURE |  |
| --- | --- | --- | --- |
|  | Check acceleration response |  | Tire position Tread depth PSI |
|  | Record shift points: |  | RF: |
|  | 1st to 2nd: MPH |  | LF: |
|  | 2nd to 3rd: MPH |  | RRO: |
|  | 3rd to 4th: MPH |  | RRI: |
|  | 4th to 5th: MPH |  | LRO: |
|  | Governed highway speed: MPH |  | LRI: |
|  | Check turning radius |  |  |
|  | Check speedometer operation |  | % BRAKE LINING: FRONT: % REAR: % |
| ADDITIONAL TASKS TO BE PERFORMED PER INSPECTION |  |  |  |
| ADDITIONAL TASKS FOR B,D,F,& H LEVEL INSPECTION |  | ADDITIONAL TASKS FOR H LEVEL INSPECTION ONLY |  |
|  | Replace transmission filter |  | Take engine oil sample |
|  | Replace coolant filter |  | Take transmission oil sample |
|  | Replace fuel filters |  | Take differential oil sample |
|  | Apply lube to all window slides and latches |  | Drain differential; install and torque plug; refill |
|  | Rotate batteries from front to rear (RTS) |  |  |
|  | Replace air filter (RTS) | ADDITIONAL TASKS FOR RTS FLEET ONLY |  |
|  |  |  | Inspect bell crank for wear, securement, and cracks |
| ADDITIONAL TASKS FOR D & H LEVEL INSPECTIONS |  |  | Inspect upper control arms for wear and securement |
|  | Drain transmission; install and torque plug; refill |  | Inspect lower control arms for wear and securement |
|  | Lubricate evaporator fanshaft bearings w/EP-2 |  | Inspect stabilizer bar for wear and damage |
|  | Replace hydraulic oil filter |  | Inspect condition and operation of engine stop cyl. |
|  | Replace air filter (Gillig) |  | Inspect condition and operation of air throttle comp. |
|  |  |  | Inspect condition and operation of fast idle cylinder |
|  | RECORD MILES ON OIL:_______________________ |  |  |
| DRIVER'S SEAT |  |  |  |
|  | Inspect foam cushions for breakdown |  | Inspect seat tilt adjustments for proper operation |
|  | Inspect seat fabric for tears or other damage |  | Inspect seat base for cracks or damage |
|  | Inspect condition of all pivot joints and bushings |  | Ensure seat is level and mounted true to column |
|  | Inspect air line for damage, pinching, or obstruction |  | Inspect air dump valve for proper operation |
|  | Check operation of slide release and lock mechanism |  | Inspect height/weight adjustment mechanism |
|  | Inspect condition of three point harness |  | Ensure full fore and aft movement on tracks |
| DEFECTS IDENTIFIED DURING INSPECTION |  |  |  |
|  | SIGNATURE OF INSPECTOR: |  | SIGNATURE OF SUPERVISOR: |
|  | DATE INSPECTION COMPLETED: |  | DATE REPAIR ORDER POSTED: |

This Page Left Blank Intentionally